|  |  |  | Maximum Input* |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Average input power | Voltage ratio dB | Maximum peak voltage | Input @ pulse width ns | Pulse Energy Joules | Risetime through unit ps | Effective bandwidth DC to | Connectors |
| 102-GP-20 | 5W | 20 | 2,500 | 400 | 0.05 | < 50 | 7 GHz | GR 874 non-locking |
| 102-GLP-20 | 5W | 20 | 2,500 | 400 | 0.05 | < 50 | 7 GHz | GR 874 locking |
| 102-NMFP-20 | 5W | 20 | 2,500 | 400 | 0.05 | < 50 | 7 GHz | N Male/Female** |
| 102 with CF Option | 10W | 20 | 2,500 | 400 | 0.05 | < 50 | 7 GHz | See available configurations above |
| 142-xxx-3 | 2W | 3 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-4 | 2W | 4 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-6B | 2W | 6 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | ** |
| 142-xxx-8B | 2W | 8 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-10B | 2W | 10 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-14B | 2W | 14 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-20B | 2W | 20 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 142-xxx-26B | 2W | 26 | 2,500 | 400 | 0.05 | < 10 | 30 GHz | *** |
| 202B-GLP-N | 2W | 8 | 5,000 | 400 | 0.20 | < 20 | 17 GHz | GR 874 locking |
| 202A-GLP-T | 2W | 14 | 5,000 | 400 | 0.20 | < 20 | 17 GHz | GR 874 locking |
| 202A-GLP-X | 2W | 20 | 5,000 | 400 | 0.20 | < 20 | 17 GHz | GR 874 locking |
| 202B-NMFP-N | 2W | 8 | 4,000 | 400 | 0.128 | < 20 | 17 GHz | N female/male |
| 202-NMFP-T | 2W | 14 | 4,000 | 400 | 0.128 | < 20 | 17 GHz | N female/male |
| 202-NMFP-X | 2W | 20 | 4,000 | 400 | 0.128 | < 20 | 17 GHz | N female/male |
| 2237A-HFNFP | 2.5W | 26 | 10,000 | 400 | 0.80 | < 50 | 7 GHz | HNB female input |
| 2239A-HFNFP | 2.5W | 26 | 16,000 | 400 | 2.05 | <100 | 3.5GHz | N female output |
| 2240A-BENFP | 2.5W | 26 | 20,000 | 200 | 1.60 | <100 | 7 GHz | Barth 454 Input N Female Output |
| 2248-HFNFP | 10W | 26 | 5,000 | 4µs | 2.00 | <100 | DC to 3.5 GHz | HN Female Input N Female Output |
| 2511-30F | 25W | 30 | 8,000 | 150 | 0.192 | < 1ns | 0.35 GHz | Fischer input BNC output |
| 2536-HFP-3 | 200W | 3 | 7,000 15,000 | 400 75 | 0.392 0.337 | 35 | 10 GHz | HNB female**** |
| 2536-HFP-6 | 200W | 6 | 7,000 15,000 | 400 75 | 0.392 0.337 | 35 | 10 GHz | HNB female**** |
| 2536-HFP-10 | 200W | 10 | 7,000 15,000 | 400 75 | 0.392 0.337 | 35 | 10 GHz | HNB female**** |
| 2536-HFP-20 | 200W | 20 | 7,000 15,000 | 400 75 | 0.392 0.337 | 35 | 10 GHz | HNB female**** |

NOTE:  Our type HN (HNB) connectors are specially designed to obtain minimum reflection coefficient for fast risetimes. For best pulse response, our model 401-HNB male or 402-HNB female cable connector for RG214/U coax should be used for interconnection.

*	Please refer to the Technical Specifications (Maximum Input Limitations) page for a full explanation of voltage and pulse width ratings.

**		Units with N connectors are limited to a 4kV rating.

***	Any male or female (GR, N, HNB, GHV) can be supplied.

****	Unit is supplied with a Barth Model 404-HMM low reflection HNB male to male adapter, so that either the input or output can be adapted to a male connection.

ZAPLESS ®