# ADVANTAGES

- High voltage pulse rated

- Least expensive high voltage coaxial attenuator

available

Model 102-NMFP-20

Small dimensions

- Higher average power than 142 series

# DESCRIPTION

# This unit utilizes a patented design, which provides for maintaining good HV capabilities and good

power dissipation.  While this unit has a simpler housing design than our precision attenuators, for

better heat dissipation, it still provides a very respectable and clean 50 ps output risetime.

# SPECIFICATIONS

# Voltage Ratio:				10.0/1 Vr (20dB)

Maximum Input:				5.0kV, 80ns FWHM Pulse, 500kW Peak Power**

2.5kV, 400ns FWHM Pulse, 125kW Peak Power

1.25kV, 1600ns FWHM Pulse, 31kW Peak Power

Average Input Power:		  	5W maximum

10W with optional fins (Model CF) for external cooling

Impedance:					50   1%

Risetime through Unit:			< 50ps

Bandwidth (-3dB):				DC to 7GHz

Reflection-TDR:				< 4% to a 100ps risetime step function

SWR:						< 1.05 to 1GHz

< 1.30 to 4GHz

Voltage Coefficient:				< 1% at rated voltage

Connectors:		102-NMFP-20		N Male/Female**

102-GLP-20		GR 874 locking

102-GP-20		GR 874 non-locking

Dimensions:		102-NMFP-20		5” long x .8” dia.

102-GP-20		4.5” long x 0.625" dia.

102-GLP-20		4.5” long x 1.01” dia.

Weight:					3/8 lb.

** 	Units with N connectors are limited to a 4kV rating.

ZAPLESS ®